Instructions for EXP

Implementation of Process synchronization algorithms using semaphore -  producer consumer problem / reader-writers problem

Bounded Buffer Problem

- Create a Menu driven program

  - Case 1 for producer

  - Case 2 for Consumer

- Create functions for wait and signal operation for simply incrementing and decrementing semaphore integer values

- Use global integer semaphore Variables for mutex, full, empty

  - Mutex=Binary semaphore

  - Full,empty=Counting Semaphore

- Initialize these variables as per Algorithm

- Take a Variable say x,  initialized to 0, to print the item number, again a global variable

Reader Writer Problem-

Take arrays for both reader and writer

The code for the writer process looks like this:

while(TRUE)

{

wait(w);

/* perform the write operation */

signal(w);

}

And, the code for the reader process looks like this:

while(TRUE)

{

//acquire lock

wait(m);

read_count++;

if(read_count == 1)

wait(w);

//release lock

signal(m);

/* perform the reading operation */

// acquire lock

wait(m);

read_count--;

if(read_count == 0)

signal(w);

// release lock

signal(m);

}